Specifications for DC700G-FT

ASSA ABLOY door closer with Cam-Motion® technology; extended mounting plate and integrated latch bolt, guide rail with integrated effeff escape door strike 332, 24V DC

- Closing force continuously adjustable, EN size 3-6

- Door closer approved in line with EN 1154, with CE mark

- Escape door strike tested in compliance with the guideline for electrical locking systems for doors along escape routes

- Closing speed, latching speed and backcheck continuously adjustable

- Suitable for fire and smoke protection doors

- Recommended door width: at least 850 mm – max. 1200 mm

- Latch position continuously adjustable between 12 mm and 17 mm

- Latch position adjustable horizontally between -3 mm and +6 mm

- Latch bolt can be adjusted vertically in 4 mm increments (-4/ 0/ +4/ +8)

- Escape door strike using FaFix, adjustable by 2 mm

- Suitable for left- and right-handed doors

- Standard installation on hinge side

Accessories

- 5 mm Spacer Plate (narrow rebate) DCFA01

- 15 mm Spacer Plate (wide rebate) DCFA02

- Replacement plate, 5 mm, for Drilling Template DCFA03

- Replacement plate, 15 mm, for External Drilling Template DCFA04

- Mounting / replacement plate, 5 mm, for Narrow Frame DCFA05

- Mounting / replacement plate, 15 mm, for Narrow Frame DCFA06

- Mechanical opening damper A188

Colours/Finishes

- Silver EV1

- Stainless steel design

Specifications for DC700G-FT BGS

ASSA ABLOY door closer with Cam-Motion® technology; extended mounting plate and integrated latch bolt, guide rail with integrated effeff escape door strike 332, 24V DC

- Closing force continuously adjustable, EN size 3-6

- Door closer approved in line with EN 1154, with CE mark

- Escape door strike tested in compliance with the guideline for electrical locking systems for doors along escape routes

- Closing speed, latching speed and backcheck continuously adjustable

- Suitable for fire and smoke protection doors

- Recommended door width: at least 850 mm – max. 1200 mm

- Latch position continuously adjustable between 12 mm and 17 mm

- Latch position adjustable horizontally between -3 mm and +6 mm

- Latch bolt can be adjusted vertically in 4 mm increments (-4/ 0/ +4/ +8)

- Escape door strike using FaFix, adjustable by 2 mm

- Suitable for left- and right-handed doors

- Standard installation on non-hinge side

- 14 mm height adjustable arm system for ease of fitting

Accessories

- Replacement plate DCFA08

- Mounting / replacement plate for Narrow Frame DCFA09

- Mechanical opening damper A188

Colours/Finishes

- Silver EV1

- Stainless steel design