Specifications for DC700G-FM

ASSA ABLOY Free-Motion door closer with Cam-Motion® technology, free-swing function starting from a door opening angle ≥ 0°, with free-swing guide rail according to EN 1155

- Closing power adjustable, EN size 3-6, for doors up to 1400 mm wide

- Barrier-free according to DIN 18040 (max. leaf width 1250 mm) and CEN/TR 15894

- Closing and latch speed adjustable from the front

- Can be used with mounting plate for fire and smoke protection doors

- CE approved

- DIN left and DIN right for door leaf mounting and frame mounting on hinge side

- Cable routing between junction box and door leaf mounting

- 14 mm height adjustable arm system for ease of fitting

Accessories

- Free-swing guide rail G795

- Mounting plate A120

- Mounting plate A130 for guide rail G795

- Smoke detector station RZ100

- Ceiling smoke detector ORS 142 / TDS 247

- Flush-mounted or surface-mounted release button

- Mechanical opening damper A153

- Connection box A189

Colours/Finishes

- Silver EV1

- White, similar to RAL9016

- Stainless steel design

- Customised finishes available on request